LS 1200d Pro USB upgrade guide

- The USB upgrade file names of LS 1200D Pro are as follows:

LS_1200d_Ctr_HardwareV?.?_SoftwareV?.?.bin

LS_1200d_Drv_HardwareV?.?_SoftwareV?.?.bin

LS_1200d_Bmp_HardwareV?.?_SoftwareV?.?.bin

The file name” LS_1200d_Ctr_HardwareV?.?_SoftwareV?.?.bin” is the firmware for the control box.

For example:

The file name” LS_1200d_Drv_HardwareV?.?_SoftwareV?.?.bin” is the firmware for the lamp head.

The file name “LS_1200d_Bmp_HardwareV?.?_SoftwareV?.?.bin” is the firmware for the control box picture data.

- Upgrade steps；

- Press the “MENU” key

- Turn the encoder knob to the upgrade option and press the "SELECT" button.

- Select the first option “Update Firmware”

- Select “Yes” and press “SELECT” button to upgrade.

- If there is no U-disk inserted, it will be prompted

- The progress bar will appear when you insert the U-disk with firmware.

- If the upgrade is successful, this interface prompt will appear.

Otherwise it will appear:

3.Note:

When upgrading separately, only put the bin file you want to upgrade into the U-disk, which can reduce the upgrade time. For example, if you just want to upgrade the control box firmware, you can only download the

.

File “LS_1200d_Ctr_HardwareV?.?_SoftwareV?.?.bin” to the U-disk.

If you want to upgrade all three or both, you can put the required firmwares into the U disk.

The order of upgrading for three of them is as follows:

LS_1200d_Drv_HardwareV?.?_SoftwareV?.?.bin

>

LS_1200d_Bmp_HardwareV?.?_SoftwareV?.?.bin

>

LS_1200d_Ctr_HardwareV?.?_SoftwareV?.?.bin

In addition, when upgrading the lamp head and picture data, the control box will not automatically turn off the screen and restart. It can only be restarted when upgrading the control box.